AP3DBL

3-Input Audio

Amplifier

Table of Contents

1. Introduction	1

1.1 Introduction to AP3DBL	1

1.2 Features	1

1.3 Package Contents	1

2. Introduction of Product Appearance	2

2.1 Introduction of Front Panel	2

2.2 Introduction of Rear Panel	3

3. System Connection	3

3.1 Usage Precautions	3

3.2 System Diagram	4

3.3 Audio Signal Connection	4

3.3.1 Audio Output	4

3.3.2 Audio Inputs	5

3.4 System Applications	5

4. System Operations	5

4.1 Operations of Front Panel	5

4.1.1 Audio switching	5

4.1.2 Volume/EQ controlling	6

4.2 Operations of IR Remote	6

4.3 Operations of Control Software	6

4.3.1 Connection with Computer	6

4.3.2 Installation/uninstallation of RS232 Control Software	7

4.3.3 Running Environment	8

4.3.4 Function Settings	8

4.3.5 RS232 Communication Commands	9

5. Specifications	11

6. Panel Drawing	12

7. Troubleshooting and Maintenance	13

8. Safety Operation Guide	13

9. Warranty	14

# Introduction

## AP3DBL Product Info

The KanexPro AP3DBL is a 3-Input 40-Watt audio amplifier supporting 70V or 100V. It comprises of 3-switchable inputs (RCA, 3.5mm and TOSLINK) for quick switching and sound reproduction. The audio amplifier supports MIC mixing and EQ control perfect for classrooms, conference rooms, lecture halls and restaurants.

Note: The unit includes IR remote & sensor. Please make sure the contents are supplied in the box or contact your reseller immediately.

## Features

- Ultra-fast switching audio amp

- Mono audio output at 40Watt.

- Switchable between 70V and 100V.

- Supports ducking function

- 16-ID codes for controlling between different AP3DBL amplifiers.

- 3-level MIC input, supports condenser microphone, dynamic microphone and wireless microphone.

- MIC port can support balance/unbalance signals & suppresses the external noise effectively.

- Two stereo audio inputs and one digital audio input, switchable by button, IR remote & RS232.

- Volume/Bass/Treble controllable by buttons, IR remote & RS232.

- Convection cooled

- LED indicator, for power and working status.

- Antistatic case design.

## Package Contents

- 1 x AP3DBL

- 1 x Power adapter (DC 24V)

- 2 x Mounting ears

- 1 x RS232 cable

- 1 x IR remote

- 1 x IR receiver

- 4 x Screws

- 1 x User manual

# Product Appearance

## Front Panel

Figure 1 Front Panel

| No. | Name | Function |
| --- | --- | --- |
| 1 | Audio Input Selection | To select the input audio source, after choosing the audio source, the corresponding LED indicator will be on. No.1 is for dual mono audio input (2 RCA connectors for L&R), No.2 is for stereo audio input (3.5mm mini jack), and No.3 is for digital fiber audio input. |
| 2 | Audio Control | Adjust the volume of the MIC, Line, or the level of Bass and Treble with this button |
| 3 | Volume Adjustment | To turn up/down or mute the corresponding audio. ▽: Turn down the volume △: Turn up the volume MUTE: Mute the output |

## Rear Panel

Figure 2e 2 Rear Panel

| No. | Name | Function |
| --- | --- | --- |
| 1 | Power Indicator | Turns red when power on. |
| 2 | Microphone input port | 3-pole captive screw connector for microphone input, the dial switch in right side is to select the micro input kind, including 48V (for condenser microphone), MIC (for dynamic microphone) and LINE (for line audio). |
| 3 | Audio Inputs | 3.5mm mini jack for stereo audio input, it can be connected with audio source device such as DVD player. |
| 4 | ID Code | 16 codes range from 0 to F (hexadecimal), works together with the PC control software. |
| 5 | IR Eye | To connect with the IR receiver, works together with the IR remote. |
| 6 | Power Port | To connect with the power adapter (DC24V). |
| 7 | 2 x RCA | Dual-mono audio input, which can be connected with audio source device such as a PC. |
| 8 | Digital Audio Input | Fiber connector for digital audio input, it can be connected with a device with fiber port, such as blue-ray player. |
| 9 | RS232 | 3-pole captive screw connector for serial control, it can be connected with PC (Use a 3-pole captive to 9 pin female D connector and serial control software) to control AP3DBL. |
| 10 | Audio Output | To connect with audio output devices, such as speakers (To select 70V or 100V depends on the input voltage of the speakers). COM is for grounding (GND). |

# System Connection

## Usage Precautions

- Make sure to connect everything before powering on.

- Speakers must be connected before powering on.

## System Diagram

Figure 3 System Diagram

## Audio Signal Connection

### Audio Output

AP3DBL supports mono audio output, and the output voltage is 70V or alterative 100V. With its dual-purpose design, it can be applied in different areas. The end COM is for grounding. The amplifier to be connected is mono audio output with a rated power at 40Watt, so AP3DBL can be connected with several speakers in parallel connection way (Total power mustn’t be more than 40Watt).

The following figure shows us how to connect with the speakers. Here we take the speakers 10Watt@8Ohms for each as example.

Figure 4 Audio Output Connection

### Audio Inputs

AP3DBL provides with 2 stereo audio inputs, one microphone input and one digital fiber audio input. The following figure shows the audio input ports.

Dial Switch, to select the MIC input mode, includes MIC (dynamic microphone), 48V (condenser microphone) and LINE (normal audio or wireless microphone).

Figure 5 Audio Input Ports

- 48V phantom power input

When the switch turns to “48V” (It has a good frequency characteristic, high input impedance and high sensitivity in this mode), the MIC input will provide a 48V phantom power. This is usually used for power supply for condenser microphone, Connection is: “+” connects to positive, “-” connects to negative and “” to ground.

Note: In this mode, only condenser microphone can be connected with.

- MIC input

When the switch turns to “MIC” (It has a low frequency characteristics, and wide frequency response in this mode), the microphone input is used for connecting with dynamic microphone. There are two different connections:

- Unbalanced connection:

“+” and “” connect to ground, and “-” connects to signal.

“-” and “” connect to ground, and “+” connects to signal.

- Balanced connection: “+” connects to positive, “-” connects to negative and “” connects to ground.

- LINE input

When the switch turns to “LINE” (It has a low frequency characteristics, and wide frequency response in this mode), the microphone input is used for connecting with normal audio or wireless microphone output. There are two different connections:

- Unbalanced connection:

“+” and “” connect to ground, and “-” connects to signal.

“-” and “” connect to ground, and “+” connects to signal.

- Balanced connection: “+” connects to positive, “-” connects to negative and “” connects to ground.

- Digital Audio Input

AP3DBL provides with a fiber optical port to connect with digital audio source device. With the SPF optical fiber, the audio signal can be transmitted faster, more stable, reliable, and can be transmitted over a long distance without distortion.

## System Applications

AP3DBL can be applied in different occasions, such as classroom, small meeting room, lecture hall, bar and hotel etc.

# System Operations

## Operations of Front Panel

The buttons provides the control of volume/EQ control and switching. The LED indicator will show the connecting status. The following content introduces audio switching and EQ control in detail.

Operations: Press the corresponding button again for cyclic switching.

### Audio switching

There are three switchable audio inputs, one 2xRCA input, one 3.5mm jack input, and one digital fiber audio input, switchable through the buttons as below:

Audio Source Selection:

1: Dual mono audio

2: Stereo audio

3: Digital fiber audio.

Figure 6 Audio Source Selection Button

### Volume/EQ controlling

The buttons can control the line volume and MIC volume.

The buttons, and controlled up/down/mute will select the MIC Volume/LINE volume/LINE bass/LINE treble by the function buttons. Please check the picture below:

Volume Adjustment:

▽: Turn down the volume

△: Turn up the volume

MUTE: Mute the output

Audio Mode Switching

Figure 7 Audio Mode and Volume Adjustment buttons

For example, to turn up the line volume, you should select the “LINE” first, and then press the button “”.

## Operations of IR Remote

AP3DBL provides with an IR eye, with the IR Receiver and the IR remote, user can control AP3DBL remotely.

Notice: The IR Receiver and the IR remote are all offered for charge.

Mute Mode:

MIC: Mute the microphone volume.

LINE: Mute the line volume.

SPEAKER: Unmute

Use to transmit the infrared signal send by the remote controller.

Audio Controlling Modes

MIC: turn up/down the microphone volume.

LINE: turn up/down the line volume.

BASS: bass tuning

TREBLE: treble of line volume.

Audio Source Selection:

1: 2 RCA dual-mono audio input

2: 1 3.5mm jack

3: 1 Digital fiber audio input

Figure 8 IR Remote

IR receiver, works together with the IR remote. Please point the IR remote at the IR receiver when use, to avoid getting out of control as there is no signal detected.

3.5mm jack

Insert it into the specialized PA3 socket (3.5mm), to connect the IR receiver with PA3.

Figure 9 IR Receivers

## Operations of Control Software

### Connection with Computer

When the amplifier connects to the COM1 or COM2 of the computer with control software, users can control it by that computer.

To control the amplifier, users should use a 3-pole male captive screw to 9-pin HD female connector and use the public COM software.

Figure 10 Connection of RS232 Port

### Installation/uninstallation of RS232 Control Software

- Installation

- Connect the input source devices and the output device according to the system diagram.

- Copy the RS232 control software to one computer, and then connect the RS232 port of this computer and AP3DBL.

- Double-click the EXE program to execute the software.

Here we take the software CommWatch.exe as example. The icon is showed as below:

Figure 11 Control Software

- Uninstallation Delete all the control software files in corresponding file path.

### Running Environment

While the control software is installed, we can activate the software through the RS232 port and set the parameters, to make it able to send RS232 commands to control AP3DBL.

### Function Settings

With the control software, we can easily switch the input channel, mute the output, check the working status, and adjust the volume etc. Please refer the details in RS232 Communication Commands.

Parameter Configuration area

Monitoring area, indicates if the command sent works.

Command Sending area

The interface of the control software is showed as below:

Figure 12 Main Interface of Control Software

### RS232 Communication Commands

Communication Protocol: RS232 Communication Protocol

Baud rate: 9600     Data bit: 8     Stop bit: 1     Parity bit: none

| Command | Function Description | Feedback Code |
| --- | --- | --- |
| 1A1. | Switching the audio to input 1 | A: 1 -> 1 |
| 2A1. | Switching the audio to input 2 | A: 2 -> 1 |
| 3A1. | Switching the audio to input 3 | A: 3 -> 1 |
| 0A0. | Mute Audio of MIC and Line out | Mute |
| 1A0. | Mute audio of MIC | Mute MIC |
| 2A0. | Mute audio of line out | Mute LIN |
| 0A1. | Unmute Audio | Unmute Audio |
| 600% | Checking the working status | A: 1 -> 1 Volume of MIC : 50 Volume of LINE : 50 Bass of LINE : 4 Treble of LINE : 4 Ducking Off |
| 601% | MIC volume up | Volume of MIC: 51 |
| 602% | MIC volume down | Volume of MIC: 51 |
| 603% | Line volume up | Volume of LINE: 51 |
| 604% | Line volume down | Volume of LINE: 51 |
| 605% | Bass level up | Bass of LINE: 4 |
| 606% | Bass level down | Bass of LINE: 4 |
| 607% | Treble level up | Treble of LINE: 4 |
| 608% | Treble level down | Treble of LINE: 4 |
| 609% | Initialization, back to the default setting | Init OK |
| 610% | Enable/disable the ducking function. | Ducking off/Ducking on |
| 4[x][x]% | Preset the volume level of ducking function. [xx] Arranges from [00] to [60]. 61 degrees in total. | Ducking of LINE: 50 |
| 5[x][x]% | Preset MIC volume, [xx] arranges from [00] to [60]. 61 degrees in total. | Volume of MIC: 50 |
| 7[x][x]% | Preset line volume, [xx] arranges from [00] to [60]. 61 degrees in total. | Volume of LINE: 50 |
| 8[x][x]% | Preset the bass level, [xx] arranges from [00] to [08]. 9 degrees in total. | Bass of LINE: 4 |
| 9[x][x]% | Preset the treble level, [xx] arranges from [00] to [08]. 9 degrees in total. | Treble of LINE: 4 |

Notice:

- The letter inside bracket [ ] is the variable code, which is changeable.

- The bracket [ ] is not included to the RS232 commands.

- Any dot “.” after the letters is part of the commands.

- Ducking function:

When input with MIC, the volume of the line audio will be automatically turned down to the preset volume level, if there is no input MIC audio signal after 5 seconds, then the volume will be automatically turned up to the original one. If you need to disable/enable the ducking function, just send the command “610%” again.

- ID coding

The ID codes of AP3DBL ranges from 0 to F (hexadecimal), when sending RS232 commands, please take notice of the address of the ID code.

If the address of the ID code is 0, any RS232 command is available.

If the address is in 1~F, it has one unique ID code (If the ID code is not the same with the address, no RS232 command will work).

While the ID code is in 1~F, please add “ID/” before sending the command.

For example, if the ID code is 5, the RS232 command needed is “604%”, the correct command is in this format: 5/604%.

There is no need to add “ID/” before the command when the ID code is 0.

Examples:

- Switching the input 2 to the line out, the command is: 2A1.

- Turning up the volume of line audio, the command is: 603%

- Preset the MIC volume to “21” degree, the command is: 521%

- Checking the working status of AP3DBL, the command is: 600%

- If the ID code is 0, sending command 601% is able to turn up the MIC volume.

If the ID code is 2, sending command 601% will not work, and the MIC volume remains unchanged. The right command is 2/601%.

# Specifications

| Audio Input |  |  | Audio Output |  |
| --- | --- | --- | --- | --- |
| Input | 2 stereo audio 1 MIC |  | Output | 1 amplifier 1 Stereo audio |
| Input Connector | 2 RCA 1 3.5mm jack 1 3-pole 3.81mm captive screw connector |  | Output Connector | 1 Captive Screw 1 x 3.5mm Jack |
| Input Impedance | >10KΩ |  | Output Type | 50Ω/stereo, 4~8Ω/Amplifier |
| Audio General |  |  |  |  |
| Frequency Response | 120Hz ~ 20KHz |  | CMRR | >70dB@20Hz~20KHz |
| SNR | 80dB (Max) |  | Bandwidth | 120Hz ~ 20KHz |
| Rated Power Output | 40Watt @8Ohms |  | THD + Noise | 1%@1KHz, 0.3%@20KHz at nominal level |
| Voltage Gain | 32dB |  |  |  |
| Control Function |  |  |  |  |
| RS232 Control | 1 3-pole 3.81mm captive screw connector |  | Front Panel Control | Buttons |
| ID Code Control | 16 ID codes for control. |  |  |  |
| Optional | IR remote & TCP/IP controlled |  |  |  |
| General |  |  |  |  |
| Temperature |  | -20 ~ +70℃ | Humidity | 10% ~ 90% |
| Power Supply |  | DC 24V power adapter | Power Consumption | 5W |
| Case Dimension |  | 1.5"x3.78"x3.45" (HWD) | Product Weight | 0.67 lbs. (0.3Kg) |

# Panel Drawing

Figure 13 Panel Drawing

# Troubleshooting and Maintenance

- When there is no output audio:

- Check if there is any signal at the input.

- Check if there is any signal at the output.

We can check these by using an oscilloscope or a multimeter. If there is no signal input/output, maybe the input/output cables broken or the connectors loosen, please change for another cable.

- Check if the output port number is the same with the controlled one.

- If not the problem mentioned above, probably there is something broken inside the unit; please contact technical support

- If the POWER indicator doesn’t work or no respond to any operation, please make sure the power cord connection is good.

- If the output sound is interfered, please make sure the system is grounded well.

- If the static becomes stronger when connecting the audio connectors, it probably due to bad grounding, please check the grounding and make sure it connected well, otherwise it would damage the amplifier.

- If the keys on the front panel, RS232 port or the IR remote, cannot control the AP3DBL amplifier the unit could be defective please contact technical support.

# Safety Operation Guide

In order to guarantee the reliable operation of the equipment and safety of the staff, please abide by the following proceeding in installation, using and maintenance:

- The system must be earthed properly. Please do not use two blades plugs and ensure the alternating power supply ranged from 100v to 240v and from 50Hz to 60Hz.

- Do not put the switcher in a place of too hot or too cold.

- As the power generating heat when running, the working environment should be maintained fine ventilation, in case of damage caused by overheat.

- Please cut off the general power switch in humid weather or left unused for long time.

- Before following operation, ensure that the alternating current wire is pull out of the power supply:

- Take off or reship any components of the equipment.

- Take off or rejoin any pin or other link of the equipment.

- As to non-professional or without permission, please DO NOT try to open the casing of the equipment, DO NOT repair it on your own, in case of accident or increasing the damage of the equipment.

- DO NOT splash any chemical substance or liquid on the equipment or around.

# Warranty

A. LIMITED WARRANTY

KanexPro TM warrants that (a) its products (the “Product”) will perform greatly in agreement with the accompanying written materials for a period of 3 years from the date of receipt and (b) that the product will be free from defects in materials and workmanship under normal use and service for a period of 3 years.

B. CUSTOMER REMEDIES

KanexPro’s entire liability and Customer’s exclusive remedy shall be, at KanexPro option, either return of the price paid for the product, or repair or replacement of the Product that does not meet this Limited Warranty and which is returned to KanexPro with a copy of customers’ receipt. This Limited Warranty is void if failure of the Product has resulted from accident, abuse, or misapplication. Any replacement Product will be warranted for the remainder of the original warranty period of 3 year, whichever is longer.

C. NO OTHER WARRANTIES

To the maximum extent permitted by applicable law, KanexPro disclaims all other warranties, either express or implied, including, but not limited to implied warranties of merchantability and fitness for a particular purpose, with regard to the product and any related written materials. This limited warranty gives customers specific legal rights. Customers may have other rights depending on the jurisdiction.

D. NO LIABILITY FOR DAMAGES

To the maximum extent permitted by applicable law, in no event shall KanexPro be liable for any damages whatsoever (including without limitation, special, incidental, consequential, or indirect damages for personal injury, loss of business profits, business interruption, loss of business information, or any other pecuniary loss) arising out of the use of or inability to use this product, even if KanexPro has been advised of the possibility of such damages.

Brea, California

KanexPro.com

MPN: AP3DBL

HDMI are trademarks or registered trademarks of HDMI Licensing LLC in the United States and other countries. KanexPro is a trademark of Apogee Inc., registered in the U.S.